University of California, San Diego

ECE 164 – Analog Integrated Circuit Design

Project Report – Fall 2025

Huayu Zhang

PID: A69040187

Email: huz028@ucsd.edu

Futong Yang

PID: A69039862

Email: fuy003@ucsd.edu

Outline of Design

1. Design Methodology

This project employs the gm/ID methodology for core circuit size design. This method avoids the inaccuracies of traditional square-law formulas in short-channel technology (0.18µm) and achieves precise control over power consumption, bandwidth, and stability.

In general, we divided the entire circuit into a bias circuit and an amplifier circuit, with two team members each responsible for one. Huayu Zhang was responsible for the amplifier circuit design, and Futong Yang for the bias circuit design. The gm/ID method played a significant role in the amplifier circuit design. We first decomposed the amplifier section into independent modules based on the function of each transistor, and then performed small signal analysis. Based on the calculation results, we could determine which parameters in the circuit directly affected various performance aspects. Using a lookup table, we could calculate the preliminary size of each transistor and estimate the required bias voltage for each node based on the project specifications. After performance testing of the amplifier circuit, the bias circuit could be designed accordingly.

For the bias circuit, first we used the ideal voltage source to bias the amplifying stages, after determining the exact DC operating points, magic battery is used to generate the same and precise gate voltage for them, which is less affected by PVT variation, only by adjusting the amounts of transistors working in triode region (or ratio k). For constant-gm circuit, the PMOS constant-gm bias circuit generates gm for PMOS differential input (same length L = 0.5μm), minimizes power consumption by current scaling, and is less affected by PVT variation. The bias core generates Iref by scaling mismatch ratio K between PMOS transistors Mb1 and Mb2. This size difference forces a specific ΔVGS across Rbias to set the reference current. Finally, adjust Rbias to control all current branches.

2. Power Optimization Strategy: Current Scaling

To minimize power dissipation while maintaining enough bandwidth and gain, we implemented a current scaling method. Instead of generating the full operating current within the bias circuit, we designed the Constant-gm reference core to operate at a very low current level of 11.3µA. This low-power reference is then mirrored to the main amplifier stages using precise ratios (scaling the MOSFET widths, W).

By decoupling the reference generation from the signal amplification, we ensured that most of the power budget is allocated to the signal path (Input Pair, Folded Cascode and Output Stage) to maximize transconductance gm, rather than being wasted in the static bias branches. This approach allowed us to achieve a total power consumption of 1.18 mW, well below the 2.5mW specification. Next, we will explain how we manually calculated and made minor adjustments to each part of the circuit and finally compare the results of the manual calculations and simulations.

Key Design Parameters Calculations

1. Hand Calculations

1.1 Two-stage amplifier circuit

1.1.1 Small-signal gain analysis

First, we analyze Stage 1. With its half-circuit model, we can approximately get:

Av1 = −Gm1 × Ro1 = − gm3 × (Rup || Rdown)

Rup ≈ gm6 ro6 (ro7 || ro3)  Rdown ≈ gm5 ro5 ro4

Similarly, for Stage 2:

Av2 = −Gm2 × Ro2 = − gm9 × (ro8 || ro9)

Therefore, the total gain:

Avtot = Av1 × Av2 = gm3 [ (gm6 ro6 (ro7 || ro3)) || (gm5 ro5 ro4) ] ∙ [ gm9 × (ro8 || ro9) ]

To achieve a gain >70dB, experience suggests that Stage 1 typically provides high gain (50~70dB), while Stage 2 provides moderate gain (20~40dB).

1.1.2 Frequency response and poles & zeros

The parameters calculated in this part are crucial for satisfying UGBW and phase margin requirements. The dominant pole is located at the gate of M9:

ωp1 ≈ −1 / Ro1 Ro2 Gm2 CC

with this equation, we can get the equation of UGBW:

UGBW ≈ Avtot × ωp1 = Gm1 / Cc = gm3 / CC

To meet UGBW ≥ 80MHz, we can easily calculate the range of gm3 / CC.

The secondary dominant pole is located at Vout, it determines the Phase Margin:

ωp2 ≈ −Gm2 / CL (1 + Cpar1 / CC) ≈ − gm9 / CL

We can achieve this approximation by setting CC to be much larger than the parasitic capacitance Cpar1 of stage 1. Considering the dominant pole, the Phase Margin = 90° − arctan(ω0dB / ωp2). In order to guarantee a PM of 65°, it is required that ωp2 ≥ 2.2 × UGBW. This determines the minimum value of gm9.

Miller compensation introduces a right half-plane zero (RHP Zero), which degrades the phase margin:

ωZ = 1 / CC [ (1 / gm9) − RC ]

By adjusting the nulling resistor RC, we can either push this zero to ∞ (RC = 1 / gm9) or let it cancel the secondary dominant pole. Our design chooses the former method.

1.1.3 Parameter calculation

a) CC: As mentioned above, we can set CC = 0.5CL = 1pF, which is large enough to suppress Cpar1 of Stage 1, while simplifying ωp2.

b) gm3: In order to meet the project spec., UGBW ≥ 80MHz, we use 90MHz for UGBW in our calculations. So gm3 = 2π × 90MHz × 1pF = 565μS ⇒ 600μS.

c) gm9: As mentioned above, we need ωp2 ≥ 2.2 × UGBW to meet the spec. for PM of 65°. To allow for adjustment, we can set gm9 = 2π × 2.5 × 90MHz × 2pF = 2.827mS ⇒ 3mS.

1.1.4 ID Budgeting

This part is the key to the gm/ID method, assigning ID based on the function of each branch.

a) Stage 1 input M3 − pursuing high gain

Considering the project spec. for Vov ≥ 150mV, gm/ID needs to be smaller than 13.3 S/A. We can set gm3/ID3 to be 12 S/A, so ID3 = gm3 / (gm3/ID3) = 600μS / 12 S/A = 50μA.Then the current of tail Itail = ID1 = ID2 = 2ID3 = 100μA.

b) Folded cascode M4 ~ M7 – consider the swing

So we set IDfold = 1.2ID3 = 60μA to prevent the transistors from slewing during signal swings. According to gm/ID = 2/Vov, we can use 200mV for Vov so that gmfold/IDfold =10 S/A.

c) Stage 2 output M8, M9 – responsible for driving load and ensuring output swing

To meet the spec. for output common-mode range 0.4~1.4V, we need to set Vov ≤ 400mV. So, with Vov = 200mV, gm9/ID9 = 2/Vov = 10 S/A.

Then the current of output branch ID8 = ID9 = gm9 / (gm9/ID9) = 3mS / 10 S/A = 300μA.

1.1.5 Sizing

In this section, we will use a lookup table in MATLAB provided by Professor Hall to find and calculate the transistor size based on the previously determined gm/ID.

First, we need to choose the L of the transistor, based on experience, choose 2x~3x Lmin. 0.5μm is a good size with Lmin = 0.18μm.

a) Stage 1 input PMOS M3: With ID3 = 50μA and gm3/ID3 = 12 S/A, we can look up the corresponding ID3/W3 = 1.2263. Then W3 = ID3 / (ID3/W3) = 40.773μm.

b) Tail PMOS M1, M2: To make sure they are in saturation with Itail = 100μA, we can set Vov = 200mV. So the gmtail/Itail = 10 S/A and corresponding Itail/Wtail = 1.9312. Then Wtail = Itail / (Itail/Wtail) = 51.781μm.

c) Folded cascode: PMOS M4,5 and NMOS M6,7

Similarly, with IDfold = 60μA, we can calculate and get W4,5 = 31.07μm & W6 = 5.766μm. As for M7, ID7 = IDfold + ID3 = 110μA, so W7 = 10.571μm.

d) Stage 2 output M8, M9: Similarly, W8 = 155.344μm and W9 = 28.84μm.

1.1.6 DC & AC analysis: check DC Operating Point and specifications

As mentioned above, the gm/ID method cannot calculate the circuit operation very accurately. Therefore, we must make necessary adjustments to the calculated parameters based on SPICE simulation results. Before building the circuit for simulation and debugging, we need to roughly determine the gate voltage bias to be provided for each transistor in the circuit. Our preliminary calculation results are as follows:

VG1 = VDD−VSG1 = 1.21V, VG2 = VDD−Vov1−VSG2 = 1.01V

VG8 = VG4 = VDD−VSG4 = 1.21V, VG5 = VDD−Vov4−VSG5 = 1.01V

Considering body effect, VG6 = Vov7+VGS6 = 0.8V ⇒ 1V.

To make sure each transistor is in sat., every Vov ≥ 150mV and to meet all of the specs. in simulations, we made the following adjustments: CC ⇒ 0.75pF, W3 ⇒ 26μm. Clearly, using the gm/ID method made our work much more efficient in this project.

1.2 Bias circuit: Constant gm bias & Magic battery

1.2.1 Constant gm bias

Rbias = 2/gm,b2 (1-1/√K)

Where K is the multiplicity and K = 4 in my project to simplify the hand calculation.

Therefore, Rbias = 1/gm,b2. Targeting a low current Iref = 10 μA to save power, assume Vov = 200mV for Mb2 so gm/ID = 10 S/A thus gm,b2 = 100μS. Finally, Rbias = 10 KΩ. Use the lookup table for current density IDp/Wp = 1.9312 then Wp = 5.18μm. Also, for the NMOS current mirror IDn/Wn = 10.4061, then Wn = 0.96μm. Use current scaling by mirrors to generate Iamp = m· Iref for amplifying stages.

1.2.2 Magic battery

Mb5a and Mb5b provide bias voltage VG2 and VG5 = VDD – 2Vov – |Vtp| with Mb6 which is around 1V. PMOS battery bias equation: VB = VDD – (k+1)1/2 Vov – |Vtp| so k = 3. The width of Mb5a is 3 times larger than Mb5b. Due to 2 branches of current flow through Mb5b, the multiplicity m = 2. The size of magic battery should be replica of amplifying stages.

In a similar way, Mb14a and Mb14b provide bias voltage VG6 = 2Vov + Vtn = 1V with Mb15. Also, the equation is VB = (k+1)1/2 Vov + Vtn so k = 3. Width of Mb14a is 3 times larger than Mb14b. Due to 2 branches of current flow through Mb14b, the multiplicity m = 2. The size of magic battery should be replica of amplifying stages.

1.3 Power Dissipation & Key Design Parameters Comparison

Itail = ID1 = ID2 = 100μA. IDfold = Ibattery = 60μA each branch. 2 folded cascode branches and 5 battery branches. Constant gm Iref = 10μA each branch and ID8 = 300μA. Power = VDD·Itot = 1.512mW (Before current scaling to minimize the power in battery stage).

| Para.; Calc.; Simu.. Iref; 10μ; 11.3µ. gm,b2; 100μ; 62.83µ. Vov; 200m; 165.9m. A0 (dB); 90.6; 76.92. UGBW; 90M; 86.9M. | Para.; Calc.; Simu.. Rbias; 10k; 18k. Cc; 1p; 0.75p. Rc; 333.3; 800. ID8; 300μ; 334.8µ. Power; 1.51m; 1.18m. | Para.; Calc.; Simu.. Pole1; 12.5k; 13.6k. Pole2; 238M; 70M. Zero; ∞; 500M. VG5; 1.01; 0.995. VG6; 1.00; 0.984. |
| --- | --- | --- |

The hand-calculation value does not fully match the simulation value. First, for gate overdrive voltage Vov, though we always assume gm/iD = 2/Vov, this is not always the case in the real design because we did not consider velocity saturation and VDS mismatch (in gm/ID method, assume VDS=0.9V), which causes channel length modulation and drain-induced barrier lowering. Those effects affect Vov (or Vgt), making the transconductance efficiency higher and affecting gm even the output impedance ro of each transistor, which leads to a large discrepancy in DC gain. Also, even if we consider body effect, it still affects the accuracy of results.

About the ratio in magic battery, for PMOS battery, its source is connected to body so no body effect theoretically, the ratio k is nearly 3 and multiplicity m = 2. However, for the NMOS battery, the real ratio k = 6, not 3 (m = 2). The body effect is one reason for it, but the main reason is the mismatch of current flows through battery and FC circuit.

Also, for the power, we changed the width of battery circuit and mirror bias. The limitation of decreasing power consumption is the “m” factor ≤ 20. Theoretically, the ratio of current in the second stage and constant-gm bias can be 203 = 8000. Similarly, current in PMOS battery can be 202 = 400 times smaller than the one in NMOS battery, which can significantly reduce the static power in biasing circuit while keeping a good gain and bandwidth.

Output Common-mode Range

Input Common-mode Range

Comments and Conclusions

Finally, we completed the project and met each required specification, but at first, we used the square law to calculate everything because we did the same thing for previous homework. Right after the first simulation for those 2 amplifying stages with ideal bias, there were too many discrepancies between the hand-calculation results and SPICE model results. Therefore, we changed our strategy – using gm/ID method to make accurate assumptions and estimations, which decreased the workload of debugging in Cadence. However, we noticed that there are still some discrepancies between gm/ID and the SPICE model. The most troubling thing is the value of Vov (Vgt), we assumed Vov (Vgt) = 200mV but it is even below 150mV after simulation because of the discrepancy between the real BSIM model and gm/id method, which drove us crazy. At last, we learned how to do more accurate start point estimation by plotting Vgt versus gm/id to make sure the actual Vgt > 150mV from a selected gm/ID.

We also achieved significant results in power consumption optimization, with total power consumption controlled at around 1.18mW, far below the upper limit of 2.5mW.

What impressed us most in this project was the exploration and use of the gm/ID method. Although there is still a gap between the manual calculation results and the actual simulation results, we have deeply appreciated the superiority of this method in improving design efficiency during the project. It greatly reduces the workload of debugging parameters in Cadence, meaning that most of the time we are clear-thinking circuit design engineers rather than spice monkeys who keep tuning parameters blindly.

Furthermore, we gained a profound understanding of the nonlinearity and highly challenging trade-offs inherent in analog circuits. The design process taught us that the parameters of analog circuits are highly coupled. For example, increasing W to improve OCMR may sacrifice GBW due to increased parasitic capacitance; reducing current to lower power consumption may cause a decrease in gm, thus worsening the phase margin. The design process is a continuous search for the "sweet points" of these trade-offs.

Overall, this project is a relatively complete design project that we completed after three months of studying analog integrated circuits. Compared to the lab problems in our regular assignments, the project is more systematic and tests a holistic view and overall thinking of an analog circuit design engineer, which will be very helpful for our future careers.